## Model Assumptions

The model is of a hypothetical hydrogen plant with PEM electrolyzers. The power for plant operation comes from an islanded wind farm with a similar capacity. It is assumed that the heat output of the electrolyzer is very large compared with the oxygen output. Therefore, the fish farm is sized first, based on the oxygen output, and the greenhouse is sized based on the heat remaining after the fish farm demand has been calculated.

Wind Farm (P_electrolyzer)

The power capacity of the wind farm for the simulation was scaled from proposed projects in Norway [1]. The capacity factor was taken from the same source and the power output over 24 hours was scaled to data from Fingrid [2]. Although Fingrid data is for all of Finland, data for specific wind farms is not easily found. Therefore, the profile is the best approximation for our project. The power output was scaled as follows:

- Hourly Averaging: The raw 3-minute real-time production data from Fingrid (March 6th-7th) was aggregated into hourly mean values to provide a consistent daily profile.

- Normalization: Each hourly mean was divided by the total national wind capacity (9,394 MW) to derive the dimensionless 'k-factor' representing the regional wind volatility.

- Calibration: Since the raw mean was 38.5 %, a multiplier of 1.16739 was applied to scale the daily average to our target 45 % Capacity Factor.

- Energy Scaling: The calibrated factors were multiplied by the farm capacity (2.9 MW) to generate the final hourly energy output (MWh).

The resulting profile ensures the total energy output over 24 hours is exactly 2.9 MW*0.45*24 hr, with no hourly peak exceeding the 2.9 MW.

Fish Farm Design

- The electrolyzer plant has a capacity of 2.5 MW; This assumption is made, as the model is vaguely based on the 2.5 MW Berlevag project [3], [4]

- The electrolyzer plant is PEM, with an efficiency of 70 %

- The wind farm has a capacity of 2.9 MW; scaled from the 175 MW electrolyzer project and assuming that the entire power output is used for hydrogen production [1]

- The wind farm has a capacity factor of 45 % [1] (The lower capacity factor from the projects in the source is selected for safety margin)

- Fish are assumed to consume 350 mg O2/kg/hr  [5], variable oxygen consumption based on feeding times is not considered (this value depends on the type of fish)

- The biomass content of the fish farm is considered to be 2.5 % [6]

To calculate the size of the fish tank, the average production of oxygen per hour must be calculated. The first step is to calculate how much energy is available for electrolysis per hour on average:

capacityFactor*1hr*capacity*eff=Eelectrolysis=0.917 MWh/hr=3.29 GJ/hr

The energy produced per hour can be divided by the formation enthalpy of one mol of hydrogen and multiplied by a factor of 0.5 to give the amount of oxygen produced per hour in mol:

Eelectrolysis∆H*0.5=O2prod=5754.97 mol/hr=184.16 kg/hr

The amount of oxygen produced per hour can be divided by the amount of oxygen required by the fish per kilogram per hour to give the amount of fish in kilograms:

O2prodO2consum=mfish=526 171.43 kg=526.17 t

This value can be used to calculate the total volume of the fish tank using the biomass content:

mfishbiomassContent*100=mtank=Vtank=21 046.86 m3

## Parameters/Stocks/Flows

| Parameter | Unit/Value | Description |
| --- | --- | --- |
| P_Electrolyzer | variable | Represents the power output of a 2.9 MW wind farm for a 2.5 MW industrial pilot scale PEM electrolyzer plant. [7] |
| Eff_Electrolyzer | 0.7 (70%) | Standard efficiency for modern alkaline or PEM electrolyzers. The remaining 30% is lost as heat, which we capture. |
| Delta_h | 285 840 J/mol | Formation enthalpy (energy required to form 1 mol of H_2). https://www.arch-goebel.ch/app/download/5817870042/Hydrogen+Production+%28Electrolysis%29-+Fundamentals+of+Water+Electrolysis%3B+Millet%3B+2015.pdf |
| M_Storage | 15000 kg | 15 tons of water. This large mass is necessary to absorb 750 kW of waste heat without causing a dangerous or rapid temperature spike |
| CP_Water | 4.18 kJ/(kg.K) | The specific heat capacity of water; used to calculate how much the temperature rises per unit of heat added. |

| Stock | Unit/Value | Description |
| --- | --- | --- |
| H2_store | mol | Represents the produced hydrogen |

| Flow | Unit/Value | Description |
| --- | --- | --- |
| H2_flow | variable | Represents the power intput 2.5 MW industrial pilot scale PEM electrolyzer plant. [7] |

## Equations

Variables

Input Power = PElectrolyzer

Efficiency = EffElectrolyzer=0.7

Formation Enthalpy = ∆H=285 840 J/mol

Hydrogen Production Rate:

Description: Converts the active electrical power (kW) into the mass flow of Hydrogen (mol/s).

H2, prod=PElectrolyzer×EffElectrolyzer∆HH2

Oxygen Production Rate:

Description: Based on the molar ratio of H2 to O2 in H2O

RateO2=RateH2×0.5

Greenhouse Temperature:

Description: It determines the temperature change in the greenhouse (∆T) per simulation interval. Q=m.cp.∆T provides the basis temperature change in greenhouse.

HeatTransfer-Qdissmgreenhouse*cpair

## Bibliography

[1]	‘PowerPoint Presentation’. Accessed: Mar. 06, 2026. [Online]. Available: https://www.hydrogen.no/files/documents/medlemsmoter/h2carrier.pdf

[2]	‘Download datasets’. Accessed: Mar. 09, 2026. [Online]. Available: https://data.fingrid.fi/en/data?datasets=181

[3]	‘NIB lends to Varanger Kraft´s hydrogen and power grid projects’, Nordic Investment Bank. Accessed: Mar. 08, 2026. [Online]. Available: https://www.nib.int/news/nib-lends-to-varanger-krafts-hydrogen-and-power-grid-projects/

[4]	‘Hydrogen-Aeolic Energy with Optimised eLectrolysers Upstream of Substation | H2020’, CORDIS | European Commission. Accessed: Mar. 08, 2026. [Online]. Available: https://cordis.europa.eu/project/id/779469/reporting

[5]	‘Dissolved oxygen requirements in aquatic animal respiration - Responsible Seafood Advocate’, Global Seafood Alliance. Accessed: Mar. 06, 2026. [Online]. Available: https://www.globalseafood.org/advocate/dissolved-oxygen-requirements-in-aquatic-animal-respiration/

[6]	‘Fish density on our farms’. Accessed: Mar. 07, 2026. [Online]. Available: https://griegseafood.com/our-impact-fish-density-on-our-farms

[7]	‘Haeolus – Hydrogen-Aeolic Energy with Optimised eLectrolysers Upstream of Substation’. Accessed: Mar. 06, 2026. [Online]. Available: https://haeolus.eu/